# Data Description

Overall:

- 12 Buses

- One file per bus

Columns in each sim_weekly_agg.csv:

- Week_Index

  - An index assigned to each week (index are independent bus-to-bus)

- Week_Start

  - Datetime to mark the beginning of the week

- Week_End

  - Datetime to mark the end of the week

- Routes

  - List of routes that the bus ran in that week

- Avg_SOC

  - Average State of Charge (on a 0-100% scale) during the week

  - What charge was the battery at

- Max_SOC [-]

  - Maximum State of Charge over the week

- Min_SOC [-]

  - Minimum State of Charge over the week

- Avg_DOD [-]

  - Depth-of-Discharge is measured as the ∆SOC from the beginning of a shift and the end of a shift (from when the bus left the depot to when it returned)

- Avg_Batt_Temp

  - Average Battery Temperature in Celsius

- Avg_Amb_Temp [degC]

  - Average Ambient Temperature in Celsius (temperature in the place where the bus is operating)

- weeklyAvgBattP

  - Weekly Average Battery Power

- weeklyDistance [km]

  - Distance the bus covered in that week

- weeklyTotDistance [km]

  - Cumulative distance the bus covers (adds each weeklyDistance week-to-week)

- weeklyDRegEn [kWh]

  - Weekly regenerated energy in kWh

- weeklyDNetEn [kWh]

  - Weekly net energy in kWh

- weeklyEnAux [kWh]

  - Energy consumed by Auxiliaries

- weeklyTotRegEn [kWh]

  - Cumulative of weeklyDRegEn week-to-week

- weeklyTotNetEn [kWh]

  - Cumulative of weeklyDNetEn week-to-week

- weeklyTotAux [kWh]

  - Total Work in kWh

- weeklyAvgCrateChg [1/h]

  - Battery charge rate

- weeklyCycles [-]

  - Number of cycles in a week

- weeklyTotCycles [-]

  - Cumulative of weeklyCycles

- weeklyDrivingTime [h]

  - Weekly Driving Time in hours

- weeklyChargingTime [h]

  - Weekly Charging Time in hours

- weeklyParkingTime [h]

  - Weekly Parking Time in hours

- Avg_Qcyc

  - Cycling aging (aging from driving) per week

  - Contributes to Avg_Qloss

- Avg_QCal

  - Calendar aging per week

  - Contributes to Avg_Qloss

- Avg_Qloss

  - This is the column to be predicted/estimated, this column represents the capacity loss (Q-loss) per week (also referred to as State of Health)